Safety Data Sheet for High Hydrogen Content Silicone Oil

Part 1: Chemical Product

Product Name: High Hydrogen Content Silicone Oil; Hydrogen-Containing Silicone Oil; Polymethylhydrosiloxane
English Name: Methylhydrogen Silicone
Chemical Name: Polymethylhydrosiloxane
CAS No.: 63148-57-2

Part 2: Hazard Overview

Physical Hazards: No hazard
Inhalation: No specific symptoms
Eye Contact: Slight irritation
Skin Contact: No specific symptoms
Ingestion: No specific symptoms
Environmental Hazards: Has certain effects on aquatic organisms
Precautions for Use: Avoid mixing with strong acids, strong alkalis, strong oxidizing agents, and compounds containing active hydrogen atoms. Keep away from open flames and high temperatures. Store in a cool, dry, and well-ventilated place.

Part 3: Composition / Information on Ingredients

Basic Information

| Chemical Name | Concentration | CAS No. | Remarks |
| --- | --- | --- | --- |
| Hydrogen-Containing Silicone Oil | Pure substance | 63148-57-2 | — |

Part 4: First-Aid Measures

Basic Information: If any symptoms occur, seek medical treatment. Contaminated clothing should be placed in a sealed container until disposal or cleaning.

Description of First-Aid Measures:
Inhalation: No special measures applicable.
Skin Contact: Remove contaminated clothing and shoes; wash with soap, detergent, and water.
Eye Contact: Immediately flush eyes with clean water for at least 15 minutes.
Ingestion: Do not induce vomiting. Rinse mouth with clean water. Seek medical attention.

Most Important Acute and Delayed Symptoms/Effects: No data
Medical Attention and Special Treatment: No data

Part 5: Fire-Fighting Measures

Basic Fire Information: Combustible; no detailed data available
Suitable Extinguishing Media: Foam and carbon dioxide
Unsuitable Extinguishing Media: Do not use direct water spray for fire extinguishing, as it may spread the flames. Do not use alkaline dry powder extinguishing agents.
Specific Hazards: Combustible liquid
Advice for Firefighters: Use water mist to cool containers. Wear self-contained breathing apparatus and full protective clothing.

Part 6: Accidental Release Measures

Emergency Measures: Extinguish all ignition sources. Keep away from alkali metals and caustic substances. Collect spilled material and prevent it from entering ditches, rivers, and the ground. Use explosion-proof electrical, ventilation, and lighting equipment. Collect this product into properly labeled containers. Containers must be tightly sealed and equipped with venting devices. Absorb residues with sand or inert materials. Rinse contaminated articles and floors with an appropriate solvent, then flush with large amounts of water. Floors may become slippery; prevent slipping.

Part 7: Handling and Storage

Precautions for Safe Handling: Maintain mechanical ventilation to prevent the formation of vapors during operation. Use electrostatic grounding equipment and suitable explosion-proof electrical equipment. The container is protected with nitrogen; ground containers and transfer equipment to eliminate static electricity. Follow good industrial hygiene practices and wash thoroughly after handling. Avoid contact with incompatible materials, and keep away from open flames and high temperatures. Store in a cool, dry, and well-ventilated place.

Part 8: Exposure Controls / Personal Protection

Requirements and Protection: Provide adequate ventilation to prevent the formation of vapors during operation. Rubber gloves, safety goggles, and suitable protective clothing are recommended. Provide eyewash facilities and showers.

Part 9: Toxicological Information

Inhalation: No data
Ingestion: No data
Skin Contact: No data
Eye Contact: No data
Acute Toxicity:
Oral: LD50 (rat) > 2000 mg/kg
Dermal: No data
Inhalation: No data

Repeated Dose Toxicity: No data
Skin Irritation and Corrosion: No data
Serious Eye Damage / Eye Irritation: No data
Respiratory or Skin Sensitization: No skin sensitization
Germ Cell Mutagenicity: In bacteria, no mutagenic components found
Carcinogenicity: No data
Reproductive Toxicity: No data

Part 10: Ecological Information

Acute Toxicity: Fish, no data. Aquatic invertebrates, no data
Chronic Toxicity: Fish, no data. Aquatic invertebrates, no data. Aquatic plants, no data
Persistence and Degradability: Not biodegradable
Mobility in Soil: No data

Part 11: Disposal Considerations

Basic Information: Refer to relevant national and local regulations before disposal
Disposal Methods: Dispose of in accordance with local laws, regulations, and product characteristics; safe incineration is recommended. Contaminated packaging should be emptied as far as possible, then recycled after cleaning or disposed of at a licensed site.

Part 12: Transport Information

This substance is not restricted by transport regulations.
Special Precautions for Users: Air transport is permitted if the packaging is fitted with a breathing valve or vent plug.

Part 13: Regulatory Information

Work Safety Law of the People’s Republic of China
Environmental Protection Law of the People’s Republic of China
Law of the People’s Republic of China on the Prevention and Control of Atmospheric Pollution
Law of the People’s Republic of China on the Prevention and Control of Water Pollution